National Textile University, Faisalabad

| Name: | Fiza Mahboob |
| --- | --- |
| Class: | BSCS(A) |
| Semester: | 5th |
| Registration No: | 23-NTU-CS-1027 |
| Course Name: | Embedded IoT Systems |
| Assignment: | 1 |
| Submitted To: | Sir Nasir |

ESP32 Multi-Mode LED Controller with OLED Display

1. Project Overview

This project implements a Multi-Mode LED Controller using an ESP32 microcontroller and an OLED display (SSD1306).
The system allows the user to cycle between different LED modes using one push button and reset all LEDs using another.

An OLED screen displays the current LED mode in real time.
The program includes debouncing logic, PWM fading, and non-blocking timing for smooth performance.

2. Project Objectives

- To control multiple LEDs using push buttons with software debouncing.

- To use OLED display for real-time feedback of LED states.

- To apply PWM (Pulse Width Modulation) for LED brightness control.

- To practice non-blocking code structure using millis() instead of delay().

- To design a complete embedded system circuit using ESP32 and external peripherals.

3. Components Used

| Component | Quantity | Description |
| --- | --- | --- |
| ESP32 Dev Board | 1 | Main microcontroller |
| Push Buttons | 2 | One for mode change, one for reset |
| LEDs | 3 | Two main LEDs and one indicator LED |
| Resistors (10kΩ) | 2 | Used as pull-down or series protection |
| OLED Display (SSD1306, I2C) | 1 | 128x64 pixel OLED for display output |
| Buzzer | 1 | Audio indication |
| Breadboard & Jumper Wires | — | For circuit connections |

4. Circuit Connections

| Component | ESP32 | Pin Description |
| --- | --- | --- |
| LED1 | GPIO 18 | Main LED 1 |
| LED2 | GPIO 17 | Main LED 2 |
| LED3 | GPIO 16 | Indicator LED for button press |
| Button 1 (Mode Btn) | GPIO 34 | Used to switch between LED modes |
| Button 2 (Reset Btn ) | GPIO 35 | Used to reset system to “BOTH OFF” |
| OLED SDA | GPIO 21 | Data line (I2C) |
| OLED SCL | GPIO 22 | Clock line (I2C) |
| Buzzer | GPIO 25 | Optional sound output |

Note: Buttons use INPUT_PULLUP, so they remain HIGH when idle and LOW when pressed.

5. Circuit Diagram (Wokwi Simulation)

You can design this circuit on Wokwi.com and export the JSON file.
Wiring color suggestions:

- Red: Power (VCC)

- Black: Ground (GND)

- Green: Data Lines (SDA/SCL)

- Yellow: Signal lines for buttons

6. Working Principle

The project operates in four LED modes, controlled by the Mode Button:

| Mode | Description | OLED Message |
| --- | --- | --- |
| 1. BOTH OFF | All LEDs remain off | BOTH OFF |
| 2. ALT BLINK | LEDs blink alternately | ALT BLINK |
| 3. BOTH ON | Both LEDs stay continuously on | BOTH ON |
| 4. PWM FADE | LEDs fade in/out using PWM | PWM FADE |

Mode Button (GPIO 34)

Each press cycles through the modes in sequence (1 → 2 → 3 → 4 → 1).

Reset Button (GPIO 35)

When pressed, it resets the mode back to “BOTH OFF”.

Indicator LED (GPIO 16)

Briefly lights up when a valid button press is detected (visual confirmation).

OLED Display (SDA=21, SCL=22)

Displays the name of the current mode in large, readable text.

7. Features Implemented

- Button debouncing using time-based logic (millis())

- Non-blocking LED control

- OLED text updates using Adafruit_SSD1306 library

- PWM fading effect for LEDs

- Mode cycling and reset logic

- Code written with meaningful variable names and clear comments

8. Source Code

#include <Arduino.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// -------- OLED Configuration ---------

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_ADDR 0x3C

// -------- Pin Definitions -----------

const int LED1_PIN = 18;

const int LED2_PIN = 17;

const int LED3_PIN = 16;

const int BTN_MODE  = 34;

const int BTN_RESET = 35;

const int BUZZER_PIN = 25;

// -------- Debounce Configuration --------

const unsigned long DEBOUNCE_DELAY = 25; // Debounce time in ms

// OLED display object

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// -------- Operating Modes -----------

enum Mode { BOTH_OFF, ALT_BLINK, BOTH_ON, PWM_FADE };

Mode currentMode = BOTH_OFF;

// ---------- Timing Variables -----------

unsigned long lastToggle = 0;       // Tracks LED blink timing

const unsigned long blinkInterval = 400; // Time interval for alternating blink

// --------- Button State Tracking -----------

unsigned long lastModePress = 0;

unsigned long lastResetPress = 0;

int lastModeState = HIGH;

int lastResetState = HIGH;

bool modePressed = false;

bool resetPressed = false;

// --------- Fade Control Variables ------------

int fadeVal = 0;

int fadeDir = 1;

// --------- Function Prototypes -------------

void setLedsToOff();

void setLedsToHigh();

void updateOLED(const char *msg);

void handleModeChange();

void setup() {

Serial.begin(115200);

// Configure pin modes

pinMode(LED1_PIN, OUTPUT);

pinMode(LED2_PIN, OUTPUT);

pinMode(LED3_PIN, OUTPUT);

pinMode(BTN_MODE, INPUT_PULLUP);

pinMode(BTN_RESET, INPUT_PULLUP);

pinMode(BUZZER_PIN, OUTPUT);

// Initialize I2C and OLED

Wire.begin(21, 22);

if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("OLED failed!");

for (;;); // Stop execution if OLED setup fails

}

// Display initial mode

updateOLED("BOTH OFF");

setLedsToOff();

Serial.println("Setup complete");

}

void loop() {

unsigned long now = millis();

// -------- MODE Button Handler --------

int readingMode = digitalRead(BTN_MODE);

// Detect change in state

if (readingMode != lastModeState) {

lastModePress = now;

}

// Debounce check

if ((now - lastModePress) > DEBOUNCE_DELAY) {

if (readingMode == LOW && !modePressed) {

modePressed = true;

digitalWrite(LED3_PIN, HIGH);

handleModeChange();

Serial.println("Mode button pressed");

}

else if (readingMode == HIGH) {

modePressed = false;

digitalWrite(LED3_PIN, LOW);

}

}

lastModeState = readingMode;

// -------- RESET Button Handler -----------

int readingReset = digitalRead(BTN_RESET);

if (readingReset != lastResetState) {

lastResetPress = now;

}

if ((now - lastResetPress) > DEBOUNCE_DELAY) {

if (readingReset == LOW && !resetPressed) {

resetPressed = true;

digitalWrite(LED3_PIN, HIGH);

currentMode = BOTH_OFF;

updateOLED("BOTH OFF");

setLedsToOff();

Serial.println("Reset button pressed");

}

else if (readingReset == HIGH) {

resetPressed = false;

digitalWrite(LED3_PIN, LOW);

}

}

lastResetState = readingReset;

// ------- LED behavior based on Mode -------

switch (currentMode) {

case BOTH_OFF:

break;

case ALT_BLINK:

// Alternate LED blinking

if (now - lastToggle >= blinkInterval) {

lastToggle = now;

static bool toggle = false;

toggle = !toggle;

analogWrite(LED1_PIN, toggle ? 255 : 0);

analogWrite(LED2_PIN, toggle ? 0 : 255);

digitalWrite(LED3_PIN, LOW);

}

break;

case BOTH_ON:

// Both LEDs stay continuously on

break;

case PWM_FADE:

// Smooth fading of both LEDs using PWM

fadeVal += fadeDir * 1;

if (fadeVal >= 180) { fadeVal = 180; fadeDir = -1; }

if (fadeVal <= 0)   { fadeVal = 0;   fadeDir = 1;  }

analogWrite(LED1_PIN, fadeVal);

analogWrite(LED2_PIN, fadeVal);

digitalWrite(LED3_PIN, LOW);

delay(20);

break;

}

}

// -------- FUNCTION DEFINITIONS ----------

// Turns off all LEDs

void setLedsToOff() {

analogWrite(LED1_PIN, 0);

analogWrite(LED2_PIN, 0);

digitalWrite(LED3_PIN, LOW);

}

// Turns on all LEDs at full brightness

void setLedsToHigh() {

analogWrite(LED1_PIN, 255);

analogWrite(LED2_PIN, 255);

digitalWrite(LED3_PIN, HIGH);

}

// Handles switching between LED operation modes

void handleModeChange() {

currentMode = (Mode)((currentMode + 1) % 4);

switch (currentMode) {

case BOTH_OFF:

updateOLED("BOTH OFF");

setLedsToOff();

break;

case ALT_BLINK:

updateOLED("ALT BLINK");

setLedsToOff();

lastToggle = millis();

break;

case BOTH_ON:

updateOLED("BOTH ON");

setLedsToHigh();

break;

case PWM_FADE:

updateOLED("PWM FADE");

fadeVal = 0;

fadeDir = 1;

break;

}

}

// Updates the OLED display with current mode information

void updateOLED(const char *msg) {

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 20);

display.println(msg);

display.display();

}

9. Code Explanation

a. Setup Section

- Initializes OLED display, sets up pins for LEDs, buttons, and buzzer.

- Starts with the “BOTH OFF” mode displayed on OLED.

b. Loop Section

- Continuously checks both buttons with debounce logic.

- Handles mode changes and reset actions.

- Executes LED behavior depending on the current mode.

c. Helper Functions

- setLedsToOff()
Turns all LEDs OFF.

- setLedsToHigh()
Turns all LEDs ON.

- handleModeChange()
Increments mode, updates OLED text, and sets LEDs accordingly.

- updateOLED(msg)
Clears screen and prints the current mode name.

10. Output Demonstration

Mode 1-Both OFF:

Mode 2- Alternate blink:

LED1-ON

LED2-ON

Mode3-Both ON:

Mode4-PWM Fade

RESET-To OFF:

11. Conclusion

This project successfully demonstrates how to integrate multiple hardware peripherals (LEDs, buttons, buzzer, and OLED) with an ESP32 board.
Through this implementation, PWM control, debouncing techniques, and I2C communication are effectively utilized.
It provides a practical understanding of real-time embedded systems and non-blocking event-driven programming.